Dual-Head Crop & Weed Perception Pipeline

Technical Report

A Comprehensive System for Agricultural Weed Perception
using Multi-Task Deep Learning, ROS 2, and RGB-D Sensing

August 2026

# 1. Introduction

Automated weed management is a critical challenge in modern precision agriculture. Manual weeding is labour-intensive, herbicide overuse degrades ecosystems, and selective mechanical weeding requires real-time, spatially accurate perception. This technical report documents the Dual-Head Crop & Weed Perception Pipeline — an end-to-end computer vision and robotics system designed to address these challenges.

The system integrates instance segmentation, plant root keypoint regression, real-time object tracking, and RGB-D depth estimation into a unified architecture. It is designed for deployment on field robots running ROS 2 (Robot Operating System 2) and embedded computing hardware such as the NVIDIA Jetson platform.

## 1.1 Motivation

Traditional weed detection approaches either rely on bounding box centre heuristics for targeting (imprecise for robotic end-effectors) or require separate, multi-model inference pipelines (computationally expensive). This project addresses both limitations by:

Predicting bounding box, class label, instance mask, and root keypoint in a single forward pass.

Providing a precise 2D root coordinate for robotic targeting, independent of bounding box geometry.

Fusing 2D root coordinates with aligned depth data to produce full 3D spatial coordinates.

## 1.2 Objectives

Develop a 4-class crop/weed classification and instance segmentation dataset from field imagery annotated in CVAT.

Design and train a dual-head YOLOv11 model that jointly predicts segmentation masks and root keypoints.

Deploy the model as a ROS 2 package with BYTETracker integration and 3D depth fusion.

Evaluate model performance using standard mAP metrics and the PCK@10 keypoint accuracy metric.

# 2. System Architecture & Pipeline Overview

The pipeline is structured as five sequential, modular stages. Figure 1 provides an overview of the end-to-end data flow from raw field annotations to real-time robotic perception.

Pipeline stages:

Stage 1: Data Preparation — CVAT XML parsing, 4-class conversion, dataset splitting.

Stage 2: Model Training — Dual-head YOLOv11, multi-task loss, ONNX export.

Stage 3: ROS 2 Deployment — ONNX inference, BYTETracker, depth fusion.

Stage 4: Web Dashboard — Real-time telemetry UI via WebSocket bridge.

Stage 5: Documentation — Technical and academic handover materials.

The key architectural innovation is the dual-head output design: a standard YOLOv11 segmentation head is augmented with a parallel root keypoint regression branch. This enables single-pass multi-task inference without running separate models.

# 3. Data Collection & Annotation Methodology

## 3.1 Taxonomic Classification Scheme

The dataset distinguishes crops and weeds based on leaf morphology into 4 target classes. This dual-category scheme allows the system to distinguish between crops to protect and weeds to destroy, while accommodating developmental stage variation within each category.

| Class ID | Class Name | Category | Description |
| --- | --- | --- | --- |
| 0 | crop_small_leaf | Crop | Seedlings / young crops with small leaf surface area |
| 1 | crop_large_leaf | Crop | Mature crops with broad, developed canopy leaves |
| 2 | weed_small_leaf | Weed | Early-stage weeds competing for nutrients |
| 3 | weed_large_leaf | Weed | Dense / mature weed clusters requiring destruction |

## 3.2 Annotation Protocol

Annotations were performed using CVAT (Computer Vision Annotation Tool) following a strict protocol:

Polygon Outer Boundary: Annotators trace the complete visible outer leaf contour for instance segmentation.

Root Keypoint Tag: A single 'points' annotation is placed at the stem root origin touching the soil plane.

Point-to-Polygon Pairing: The conversion script (02_convert_xml_to_yolo.py) automatically pairs each root keypoint with its enclosing plant polygon.

## 3.3 YOLO Label Format

Each annotated object is stored in a YOLO-format .txt label file. The normalized line format encodes the class ID, followed by the root keypoint coordinate, followed by all polygon vertices:

class_id  x_root  y_root  x1  y1  x2  y2  ...  xN  yN

All coordinates are normalized relative to image dimensions (width W, height H), such that x_norm = x / W and y_norm = y / H. This normalization ensures scale-invariance across varying camera resolutions.

## 3.4 Dataset Split

| Split | Proportion | Purpose |
| --- | --- | --- |
| Train | 80% | Model parameter optimization |
| Validation | 5% | Hyperparameter tuning and early stopping |
| Test | 15% | Final held-out evaluation |

# 4. Model Architecture

## 4.1 Backbone & Neck

The model builds upon the YOLOv11 Medium (YOLOv11-M) backbone, which produces multi-scale feature maps at three pyramid levels (P3, P4, P5). A Feature Pyramid Network (FPN) and Path Aggregation Network (PAN) neck fuses these features to capture objects at multiple scales.

## 4.2 Dual-Head Output Design

The standard YOLOv11 segmentation head is augmented with a fourth parallel branch for root keypoint regression. The four parallel output branches are:

cv2 — Bounding box regression (DFL encoding + CIoU loss)

cv3 — Classification logits (4 classes, BCE loss)

cv4 — Mask prototype coefficients (32 channels, BCE prototype loss)

cv5 — Root keypoint regression (2 channels: x_root, y_root, L1 loss)  ★ Novel addition

This architecture enables a single forward pass to simultaneously output all required information for crop/weed detection, segmentation, and robotic targeting.

## 4.3 ONNX Export

After training, the model is exported to ONNX (Open Neural Network Exchange) format using the export_onnx.py script. ONNX Runtime enables efficient, hardware-accelerated inference on deployment hardware (x86 PCs, NVIDIA Jetson) without requiring a PyTorch environment.

# 5. Loss Functions & Training Setup

## 5.1 Multi-Task Loss Formulation

The total training objective is a weighted linear combination of five loss components:

L_total = λ_box · L_box  +  λ_seg · L_seg  +  λ_cls · L_cls  +  λ_dfl · L_dfl  +  λ_kpt · L_kpt

| Component | Symbol | Type | Purpose |
| --- | --- | --- | --- |
| Bounding box | L_box | CIoU Loss | Penalizes localization error |
| Segmentation | L_seg | BCE Prototype Loss | Penalizes mask prediction error |
| Classification | L_cls | BCE Loss | Penalizes class probability error |
| Distribution | L_dfl | Distribution Focal Loss | Sharpens bounding box distribution |
| Keypoint | L_kpt | L1 Loss | Penalizes root coordinate error |

## 5.2 Root Keypoint Regression Loss

For foreground matched anchors (the set N_fg), the keypoint loss minimizes the L1 distance between predicted root coordinates (x_hat, y_hat) and ground-truth (x, y):

L_kpt = (1 / |N_fg|) · Σ_{i ∈ N_fg} ( |x_hat_i - x_i| + |y_hat_i - y_i| )

The L1 loss is preferred over L2 (MSE) for keypoint regression because it is less sensitive to outlier annotations and produces more stable gradients for sparse keypoint targets.

## 5.3 PCK@10 Evaluation Metric

Keypoint localization accuracy is evaluated using the Percentage of Correct Keypoints (PCK) metric, normalized by the bounding box max dimension S = max(W_box, H_box):

PCK@α = (1/N) · Σ_{i=1}^{N} 𝟙 [ d(k_i, k_hat_i) / S_i ≤ α ]

where α = 0.10 for PCK@10. A keypoint prediction is considered correct if its Euclidean distance from the ground-truth is within 10% of the plant's bounding box size. This scale-normalised metric accounts for varying plant sizes across the dataset.

# 6. ROS 2 Deployment & Integration

## 6.1 Node Architecture

The plant_perception ROS 2 package implements a single processing node (plant_perception_node) that executes the full inference pipeline in real time. The node processes synchronized RGB and aligned depth frames from an RGB-D camera.

Processing pipeline steps:

Approximate Time Synchronization: Synchronizes /camera/color (RGB) and /camera/depth (16-bit) frames.

Letterbox Preprocessing: Resizes the input frame to the model's input resolution while preserving aspect ratio.

ONNX Forward Execution: Runs the exported model through ONNX Runtime for inference.

NMS & Keypoint Rescaling: Applies Non-Maximum Suppression and maps root keypoints back to native camera resolution.

BYTETracker ID Assignment: Associates detections across consecutive frames using Kalman filtering and IoU matching, assigning persistent track IDs.

Median Depth Sampling: Samples depth in a 7×7 pixel window around the root keypoint to produce a robust 3D coordinate.

Message Publishing: Publishes full plant telemetry to /plant_perception/tracked_plants.

## 6.2 3D Root Coordinate Estimation

A 7×7 median filter is applied to the depth image around the predicted root pixel coordinate (x_root, y_root) to compute a robust depth estimate:

Z_root = median({ D(x,y) | (x,y) ∈ W_7×7(x_root, y_root), D(x,y) > 0 })

The median is computed only over valid (non-zero) depth pixels to exclude sensor dropout regions. The resulting depth Z_root combined with the 2D coordinate (x_root, y_root) and camera intrinsic parameters yields the full 3D root position in the camera frame.

## 6.3 ROS Message Definition

Each tracked plant is published as a TrackedPlant.msg with the following fields:

| Field | Type | Description |
| --- | --- | --- |
| header | std_msgs/Header | Timestamp and frame ID |
| track_id | int32 | Persistent BYTETracker plant ID |
| class_id | int32 | 4-class category index (0–3) |
| class_name | string | Human-readable class label |
| confidence | float32 | Detection confidence score |
| x1,y1,x2,y2 | float32 (×4) | Bounding box pixel coordinates |
| root_x, root_y | float32 (×2) | Root keypoint 2D pixel position |
| root_d | float32 | Median sampled depth at root point |
| mask | sensor_msgs/Image | Binary instance segmentation mask |

# 7. Conclusion

This report has presented the complete design and implementation of the Dual-Head Crop & Weed Perception Pipeline — a unified, modular system for real-time agricultural plant perception on field robots.

Key contributions of this project include:

A 4-class annotated dataset with paired polygon masks and root keypoint annotations created using CVAT.

A novel dual-head YOLOv11 architecture that simultaneously predicts detection, segmentation, and root keypoints in a single forward pass.

A multi-task loss function combining CIoU, BCE, DFL, and L1 losses for joint optimization.

A PCK@10 evaluation framework for quantitative root keypoint accuracy assessment.

A complete ROS 2 deployment package with BYTETracker, RGB-D depth fusion, and real-time ROS messaging.

## 7.1 Future Work

Expand the dataset to include more crop species and weed varieties from different field conditions.

Integrate 3D point cloud generation from fused depth data for volumetric plant modelling.

Benchmark model latency on NVIDIA Jetson Orin embedded hardware.

Develop a closed-loop control interface between the ROS 2 perception node and a robotic arm end-effector.